REQUIRED INSPECTIONS

Footings and Setbacks


After footing forms and reinforcing steel have been placed, prior to concrete being poured.

Foundation Walls

After foundation wall forms and reinforcing steel have been placed, prior to concrete being poured.
Concrete Slabs

Any slabs containing thickened slab areas, for integral footings at bearing walls and point loads are to be inspected after grading, placement of forms and reinforcing steel, prior to concrete being poured
Vapor retarder required between graded compacted fill and concrete slabs at heated/conditioned spaces.  Insulation required around perimeter of slabs on grade and under radiant slabs.  

*See the Summary of the Washington State Energy Code handout for specific R-value requirements and methods.

*Non-structural slabs are not required to be inspected.

Plumbing

In-ground/under slab plumbing: to be inspected after waste, vent and supply piping have been placed, before ditches and trenches have been backfilled.

Rough-in plumbing: to be inspected after installation of all waste, vent and supply piping, prior to or with framing inspection.


*All plumbing piping and systems to be tested and inspected.


Waste and vent piping – water tested with 10’ water column, above highest fitting,


or 5# of air, for 15 minutes.


Supply piping – water tested at working pressure with potable water,


or not less than 50# of air.

Sewer as Built

After the building sewer piping has been installed from the building, and connected to sewer tap, before pipe is covered and trench backfilled.

Septic


All septic perk testing, design and engineering, inspections and permitting performed by 

Asotin County Health Department 758-3344

HVAC

After all ductwork, equipment, appliances and venting have been installed, prior to or with framing inspection.  

Natural Gas Piping.

*A separate mechanical/gas-piping permit is required. Obtainable through Asotin County Building & Planning Department, Asotin County will also be doing inspections as of 4/01/06..
REQUIRED INSPECTIONS

(Continued)

Electrical

Electrical permits and inspections shall be obtained and performed by the Washington State Department of Labor and Industries, Electrical Division, Pullman, Wa.1-800-509-0025 

*Check with D.L&I for required inspections.
*Electrical final inspection will be required before final inspection and issuance of Certificate of Occupancy.
Framing

After all framing, sheathing, roofing, bracing, fire blocking, draft stops and sealing of penetrations are complete.  All ductwork for exhaust fans to be vented to the outside, installation of fireplace chimney, ductwork and direct venting to be completed.  

After all rough-in plumbing, HVAC, gas piping and electrical cover inspections have been completed, prior to covering or insulating.

*Provide floor designs at job site (engineered if required).

*Provide engineered roof truss system design at job site.

*Provide installation instructions and specifications for fireplaces and other specialty gas 

  appliances at job site.

Insulation

After all insulation is installed in wall cavities and concealed spaces and vapor barrier installed (if visqueen is used), prior to sheetrock.

*See the Summary of the Washington State Energy Code handout for specific R-value 

  requirements and methods.

Sheetrock 

Drywall nailing and screwing to be inspected prior to taping and finishing.

Final Inspection and Issuance of Certificate of Occupancy

The final inspection is to be performed after all work is completed and the building is ready for occupancy.

*Electrical final inspection and House #’s attached required prior to final inspection & issuance of Certificate of Occupancy  
See Additional Handouts and Information:

Available at the Asotin County Building Department front counter, for other specific requirements and details to complete your building project.


Asotin County Structural Load Requirements


Typical Site Plan and Asotin County Zoning Setbacks


Crawl Space Cross Section


8 Foot Concrete Wall Cross Section


Slab on Grade Cross Section


Monolithic Slab Section


Pole Building Cross Section


Foundation Clearances from Slopes


Emergency Escape and Rescue Openings


Hazardous Glazing Locations


Residential Stairways, Guards and Handrails

Summary of the Washington State Energy Code
