BACKPACK SPRAYER CALIBRATION

NO MATH VERSION

Step 1. Measure and mark a calibration plot that is exactly

18.5 feet wide X 18.5 feet long

Step 2. Spray the calibration plot uniformly with water, noting the number of seconds required, do this three times and average. Spray at your normal or usual pace.

Time Required = _____________Seconds

Step 3. Spray into a bucket for the same number of seconds.

Step 4. Measure the number of ounces of water in the bucket.

Volume Sprayed = _____________ Ounces

Step 5. The number of ounces collected from the bucket is equal to the number of gallons per acre the sprayer is delivering.

Gallons Per Acre (GPA) = ____________________

CORRECT AMOUNT OF HERBICIDE PER TANK

FOR LIQUID HERBICIDE FORMULATIONS

STEP 1: Record sprayer output in gallons/acre (from step 5 calibration sheet)

Output (Volume) = _____________GPA

STEP 2: Determine volume of full spray tank.

Tank volume = ______________gallons

STEP 3: From the herbicide label determine the amount of herbicide concentrate to apply per acre.

___________Herbicide per acre, oz, pts, qts.

| STEP 4: Determine the amount of herbicide to add for each gallon of water in the sprayer from the chart below. Spray Volume GPA | Amount of Herbicide to Add To Each Gallon |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Recommended Herbicide Rate Per Acre |  |  |  |  |  |  |  |  |
|  | 1 Pint | 1 Quart |  | 2 quarts |  | 3 quarts |  | 4 quarts |  |
| 15 | 6tsp |  | 2 fl/oz |  | 4 fl/oz |  | 6.25 fl/oz |  | 6.50 fl/oz |  |
| 20 | 5tsp |  | 10 tsp |  | 3.25 fl/oz |  | 4.75 fl/oz |  | 6.33 fl/oz |  |
| 30 | 3tsp |  | 6 tsp |  | 2 fl/oz |  | 3.25 fl/oz |  | 4.25 fl/oz |  |
| 40 | 2.33 tsp |  | 4.75 tsp |  | 1.66 fl/oz |  | 2.33 fl/oz |  | 3.25 fl/oz |  |
| 50 | 2 tsp |  | 3.75 tsp |  | 1.25 tsp |  | 2 fl/oz |  | 2.50 fl/oz |  |
| 60 | 1.66 tsp |  | 3.25 tsp |  | 6.33 tsp |  | 1.66 fl/oz |  | 2 fl/oz |  |
| 70 | 1.33 tsp |  | 2.75 tsp |  | 5.50 tsp |  | 1.33 fl/oz |  | 1.75 fl/oz |  |
| 80 | 1.25 tsp |  | 2.33 tsp |  | 4.75 tsp |  | 7.25 tsp |  | 9.50 tsp |  |
| 90 | 1 tsp |  | 2 tsp |  | 4.25 tsp |  | 6.33 tsp |  | 8.50 tsp |  |
| 100 | 1 tsp |  | 2 tsp |  | 3.75 tsp |  | 5.75 tsp |  | 7.66 tsp |  |
| 120 | 0.75 tsp |  | 1.50 tsp |  | 3 tsp |  | 4.75 tsp |  | 6 tsp |  |

STEP 5: Calculate the amount of herbicide to add to each tank.

______ Amount of herbicide/gallon X ________ number of gallons in tank

= _________ Total amount of herbicide to add to each tank load.

Example: You calibrate your sprayer and the output is 30 GP A, and your sprayer holds 3 gallons. You are spraying spotted knapweed and want an herbicide application rate of I pint/acre. Go to the

chart and read across from 30 GallA - the amount of herbicide to add per gallon is 3 tsp.. Since your

sprayer holds 3 gallons of total solution you would add 9 tsp (3 TBS) of herbicide in addition to the water to each tank.

Liquid conversions: tsp = teaspoons; TBS = tablespoons; fl oz = fluid ounces, 3 teaspoons = 1 tablespoon, 8 fluid ounces = 1 cup,

2 tablespoon = 1 fluid ounce, 1 cup = 16 tablespoons.

CORRECT AMOUNT OF HERBICIDE PER TANK

FOR ESCORT DISPERSIBLE GRANULE HERBICIDE

STEP 1: Record sprayer output in gallons/acre (from step 5 calibration sheet)

Output (Volume) = _____________GPA

STEP 2: Determine volume of full spray tank.

Tank volume = ______________gallons

STEP 3: From the herbicide label determine the amount of herbicide to apply per acre.

___________Herbicide per acre, oz, pts, qts.

| STEP 4: Determine the amount of herbicide to add for each gallon of water in the sprayer from the chart below. Escort gram tube conversion table |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Escort rate in grams per gallon |  |  |  |  |  |  |  |
| Herbicide Rate/acre | Grams / 1gal |  | Grams / 2gal |  | Grams / 3gal |  | Grams / 4gal |
| ½ oz |  | ¼ |  | ½ |  | ¾ |  |
| 1 oz | ¼ |  | ½ |  | ¾ |  | 1 |
| 2 oz | ½ |  | 1 |  | 1¾ |  | 2¼ |